Floor Plan Measurement - wireless survey

Wireless site survey and design for Contoso Ltd

| Item | Detail |
| --- | --- |
| Client | Contoso Ltd |
| Site | Floor Plan Measurement - office floor |
| Survey type | Predictive (desktop model) |
| Survey date | 21 September 2026 |
| Document reference | NSS-2026-042 |
| Revision | A |
| Status | For client review |
| Prepared by | A. Engineer |
| Issued | 2026-09-21 |
| Prepared for | Contoso Ltd |
| Prepared by (company) | Niyyasoft |

Commercial in confidence. This report is issued to Contoso Ltd for the purpose of the works described in it, and should not be passed to a third party without written agreement. Predicted coverage is a model of the drawings supplied and the materials recorded during the survey; it is not a measurement of the finished installation.

# Contents

The contents build when the document opens. If this line is still showing, select all and press F9 to update it.

# 1. Executive summary

- 22 access points are proposed across 2 floor plan(s), covering 2,097 m2 - about one access point per 95.3 m2.

- On 5 GHz the design puts 100 % of the surveyed area at or above the -67 dBm target, with 95.7 % reached by two or more access points for resilience and roaming.

- The hardware listed in the bill of materials draws 841.5 W of Power over Ethernet across 22 switch ports.

- The switching proposed cannot currently power it: the PoE budget is short by 404.5 W. This must be resolved before installation.

## In the surveyor's words

Eleven Wi-Fi 6E access points cover 100 % of the 1,302 m2 floor plate at or above -67 dBm.

## Matters needing attention

| Severity | Where | What we found | What we recommend |
| --- | --- | --- | --- |
| Medium | Service core | The concrete core sits in the middle of the floor plate and blocks the direct path between the north and south halves. | Keep APs on both sides of the core - the layout in this report already does. |

## Design checks

An automatic review of this design against the figures in the rest of the report.

| Level | Finding |
| --- | --- |
| Action required | PoE budget is short by 404.5 W: 841.5 W of demand against 437 W available. |
| Review | "Level 1 general arrangement" has 11 radios on 2.4 GHz, where only three channels do not overlap. The closest pair sharing a channel is 0 m apart. Consider turning 2.4 GHz off on roughly every second access point. |
| Review | "Floor Plan Measurement" has 11 radios on 2.4 GHz, where only three channels do not overlap. The closest pair sharing a channel is 12.2 m apart. Consider turning 2.4 GHz off on roughly every second access point. |
| Review | The survey report is 16 % complete (16 of 101 fields). Fill it in before issuing this to a client. |
| For information | No company logos are attached, so the report will be issued unbranded. |

# 2. Document control

| Item | Detail |
| --- | --- |
| Report reference | NSS-2026-042 |
| Revision | A |
| Status | For client review |
| Prepared by | A. Engineer |

# 3. Site and scope

| Item | Detail |
| --- | --- |
| Survey type | Predictive (desktop model) |
| Survey date(s) | 21 September 2026 |
| Site address | Office floor plate, 42 m frontage |
| Building type | Office |
| Survey equipment used | Professional survey software, Laser distance meter |

# 4. Predicted coverage

## How to read the heat maps

Each heat map shows the signal strength a typical client device should see at floor level, in dBm. dBm is a negative number: the closer to zero, the stronger the signal. -45 dBm is very strong, -67 dBm is the design target for reliable voice and video, and -80 dBm is too weak to be useful.

| Colour | Signal | What it means in practice |
| --- | --- | --- |
| Dark green | Stronger than -55 dBm | Full performance, close to an access point |
| Green | -55 to -62 dBm | Comfortable for everything including video calls |
| Yellow | -62 to -67 dBm | Meets the design target; voice and video remain reliable |
| Orange | -67 to -72 dBm | Usable for data, marginal for voice |
| Red | Weaker than -72 dBm | Below target - expect slow or dropped connections |
| No colour | No usable signal | Outside the surveyed area, or fully shielded |

Two or more access points reaching a point matters as much as raw signal: it is what lets a device roam without dropping a call, and what keeps an area covered if one access point fails. The "two or more APs" column below is that figure.

## Coverage against target

| Floor | Band | At or above target | At or above secondary | Two or more APs | Mean signal | Weakest point | Mean SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Level 1 general arrangement | 2.4 GHz | 97.8 % | 98.6 % | 97.5 % | -45.1 dBm | -77 dBm | 29.1 dB |
| Level 1 general arrangement | 5 GHz | 97.5 % | 97.6 % | 96.9 % | -45.7 dBm | -85.4 dBm | 34 dB |
| Level 1 general arrangement | 6 GHz | 97.5 % | 97.5 % | 93.1 % | -47.3 dBm | -91.8 dBm | 32.7 dB |
| Floor Plan Measurement | 2.4 GHz | 100 % | 100 % | 97.7 % | -43.6 dBm | -58.2 dBm | 20.2 dB |
| Floor Plan Measurement | 5 GHz | 100 % | 100 % | 95.7 % | -43.7 dBm | -57.8 dBm | 35 dB |

## Floors surveyed

| Floor | Area | Access points | Area per AP | Walls modelled | Scale |
| --- | --- | --- | --- | --- | --- |
| Level 1 general arrangement | 795 m2 | 11 | 72 m2 | 16 | 59.1 px/m (image resolution) |
| Floor Plan Measurement | 1,302 m2 | 11 | 118 m2 | 6 | 8.0 px/m (measured line on the drawing) |

## Heat map - Level 1 general arrangement

Predicted signal strength. predictive coverage

## Heat map - Floor Plan Measurement

Predicted signal strength. predictive coverage

# 5. Access point schedule

Where each access point goes, how it is mounted, and which switch port feeds it. Positions are measured from the top-left corner of the drawing.

| Floor | AP | Model | Mounting | Position | Switch / port | IP |
| --- | --- | --- | --- | --- | --- | --- |
| Level 1 general arrangement | AP-01 | Cisco Catalyst CW9166I | ceiling @ 2.7 m | 8.43, 6.5 m | SW-L1-A Gi1/0/1 | 10.10.99.11 |
| Level 1 general arrangement | AP-02 | Cisco Catalyst CW9166I | ceiling @ 2.7 m | 21.99, 6.5 m | SW-L1-A Gi1/0/2 | 10.10.99.12 |
| Level 1 general arrangement | AP-03 | Cisco Catalyst CW9166I | ceiling @ 2.7 m | 36.66, 6.5 m | SW-L1-A Gi1/0/3 | 10.10.99.13 |
| Level 1 general arrangement | AP-04 | Cisco Catalyst CW9166I | ceiling @ 2.7 m | 11.01, 19.49 m | SW-L1-A Gi1/0/4 | 10.10.99.14 |
| Level 1 general arrangement | AP-05 | Cisco Catalyst CW9166I | ceiling @ 2.7 m | 25.67, 19.49 m | SW-L1-A Gi1/0/5 | 10.10.99.15 |
| Level 1 general arrangement | AP-06 | Cisco Catalyst CW9166I | ceiling @ 2.7 m | 40.33, 19.49 m | SW-L1-A Gi1/0/6 | 10.10.99.16 |
| Level 1 general arrangement | AP-07 | Cisco Catalyst CW9166I | ceiling @ 2.8 m | 13.19, 7.81 m |  |  |
| Level 1 general arrangement | AP-08 | Cisco Catalyst CW9166I | ceiling @ 2.8 m | 8.8, 6.5 m |  |  |
| Level 1 general arrangement | AP-09 | Cisco Catalyst CW9166I | ceiling @ 2.8 m | 8.8, 6.5 m |  |  |
| Level 1 general arrangement | AP-10 | Cisco Catalyst CW9166I | ceiling @ 2.8 m | 12.31, 6.5 m |  |  |
| Level 1 general arrangement | AP-11 | Cisco Catalyst CW9166I | ceiling @ 2.8 m | 15.83, 6.5 m |  |  |
| Floor Plan Measurement | AP-01 | Cisco Catalyst 9120AXI | ceiling @ 2.8 m | 28.34, 14.79 m | SW-A Gi1/0/1 | 10.10.99.20 |
| Floor Plan Measurement | AP-02 | Cisco Catalyst 9120AXI | ceiling @ 2.8 m | 40.25, 14.79 m | SW-A Gi1/0/2 | 10.10.99.21 |
| Floor Plan Measurement | AP-03 | Cisco Catalyst 9120AXI | ceiling @ 2.8 m | 52.28, 14.79 m | SW-A Gi1/0/3 | 10.10.99.22 |
| Floor Plan Measurement | AP-04 | Cisco Catalyst 9120AXI | ceiling @ 2.8 m | 19.31, 23.07 m | SW-A Gi1/0/4 | 10.10.99.23 |
| Floor Plan Measurement | AP-05 | Cisco Catalyst 9120AXI | ceiling @ 2.8 m | 31.34, 23.07 m | SW-A Gi1/0/5 | 10.10.99.24 |
| Floor Plan Measurement | AP-06 | Cisco Catalyst 9120AXI | ceiling @ 2.8 m | 43.25, 23.07 m | SW-A Gi1/0/6 | 10.10.99.25 |
| Floor Plan Measurement | AP-07 | Cisco Catalyst 9120AXI | ceiling @ 2.8 m | 55.29, 23.07 m | SW-A Gi1/0/7 | 10.10.99.26 |
| Floor Plan Measurement | AP-08 | Cisco Catalyst 9120AXI | ceiling @ 2.8 m | 16.3, 31.47 m | SW-A Gi1/0/8 | 10.10.99.27 |
| Floor Plan Measurement | AP-09 | Cisco Catalyst 9120AXI | ceiling @ 2.8 m | 28.34, 31.47 m | SW-A Gi1/0/9 | 10.10.99.28 |
| Floor Plan Measurement | AP-10 | Cisco Catalyst 9120AXI | ceiling @ 2.8 m | 40.25, 31.47 m | SW-A Gi1/0/10 | 10.10.99.29 |
| Floor Plan Measurement | AP-11 | Cisco Catalyst 9120AXI | ceiling @ 2.8 m | 52.28, 31.47 m | SW-A Gi1/0/11 | 10.10.99.30 |

## Channel and power plan

Channels are planned so that access points sharing one are as far apart as the floor allows. Transmit power is set for the design, not the maximum the radio supports.

| Floor | AP | 2.4 GHz | 5 GHz | 6 GHz |
| --- | --- | --- | --- | --- |
| Level 1 general arrangement | AP-01 | ch 1 / 20 MHz / 11 dBm | ch 36 / 80 MHz / 17 dBm | ch 5 / 160 MHz / 17 dBm |
| Level 1 general arrangement | AP-02 | ch 6 / 20 MHz / 11 dBm | ch 52 / 80 MHz / 17 dBm | ch 37 / 160 MHz / 17 dBm |
| Level 1 general arrangement | AP-03 | ch 11 / 20 MHz / 11 dBm | ch 100 / 80 MHz / 17 dBm | ch 69 / 160 MHz / 17 dBm |
| Level 1 general arrangement | AP-04 | ch 11 / 20 MHz / 11 dBm | ch 116 / 80 MHz / 17 dBm | ch 101 / 160 MHz / 17 dBm |
| Level 1 general arrangement | AP-05 | ch 1 / 20 MHz / 11 dBm | ch 132 / 80 MHz / 17 dBm | ch 133 / 160 MHz / 17 dBm |
| Level 1 general arrangement | AP-06 | ch 6 / 20 MHz / 11 dBm | ch 149 / 80 MHz / 17 dBm | ch 165 / 160 MHz / 17 dBm |
| Level 1 general arrangement | AP-07 | ch 1 / 20 MHz / 11 dBm | ch 36 / 80 MHz / 17 dBm | ch 5 / 160 MHz / 17 dBm |
| Level 1 general arrangement | AP-08 | ch 1 / 20 MHz / 11 dBm | ch 36 / 80 MHz / 17 dBm | ch 5 / 160 MHz / 17 dBm |
| Level 1 general arrangement | AP-09 | ch 1 / 20 MHz / 11 dBm | ch 36 / 80 MHz / 17 dBm | ch 5 / 160 MHz / 17 dBm |
| Level 1 general arrangement | AP-10 | ch 1 / 20 MHz / 11 dBm | ch 36 / 80 MHz / 17 dBm | ch 5 / 160 MHz / 17 dBm |
| Level 1 general arrangement | AP-11 | ch 1 / 20 MHz / 11 dBm | ch 36 / 80 MHz / 17 dBm | ch 5 / 160 MHz / 17 dBm |
| Floor Plan Measurement | AP-01 | ch 1 / 20 MHz / 11 dBm | ch 36 / 80 MHz / 17 dBm | - |
| Floor Plan Measurement | AP-02 | ch 6 / 20 MHz / 11 dBm | ch 52 / 80 MHz / 17 dBm | - |
| Floor Plan Measurement | AP-03 | ch 11 / 20 MHz / 11 dBm | ch 100 / 80 MHz / 17 dBm | - |
| Floor Plan Measurement | AP-04 | ch 11 / 20 MHz / 11 dBm | ch 116 / 80 MHz / 17 dBm | - |
| Floor Plan Measurement | AP-05 | ch 6 / 20 MHz / 11 dBm | ch 132 / 80 MHz / 17 dBm | - |
| Floor Plan Measurement | AP-06 | ch 1 / 20 MHz / 11 dBm | ch 149 / 80 MHz / 17 dBm | - |
| Floor Plan Measurement | AP-07 | ch 6 / 20 MHz / 11 dBm | ch 116 / 80 MHz / 17 dBm | - |
| Floor Plan Measurement | AP-08 | ch 1 / 20 MHz / 11 dBm | ch 100 / 80 MHz / 17 dBm | - |
| Floor Plan Measurement | AP-09 | ch 11 / 20 MHz / 11 dBm | ch 52 / 80 MHz / 17 dBm | - |
| Floor Plan Measurement | AP-10 | ch 6 / 20 MHz / 11 dBm | ch 36 / 80 MHz / 17 dBm | - |
| Floor Plan Measurement | AP-11 | ch 11 / 20 MHz / 11 dBm | ch 132 / 80 MHz / 17 dBm | - |

# 6. Bill of materials

| Vendor | SKU | Model | Category | Qty | PoE class |
| --- | --- | --- | --- | --- | --- |
| Cisco | C9120AXI | Catalyst 9120AXI | Access point | 11 | 802.3at |
| Cisco | C9300-48P | Catalyst 9300-48P | Access switch | 1 | 802.3at |
| Cisco | C9800-L-C-K9 | Catalyst 9800-L (Copper) | Controller / gateway | 1 | - |
| Cisco | CW9166I | Catalyst CW9166I | Access point | 11 | 802.3bt |

Access ports required: 22. Switch ports provided: 48 (headroom 26). PoE demand: 841.5 W against 437 W of switch budget.

Action required - PoE budget is short by 404.5 W: 841.5 W of demand against 437 W available. Add switch capacity or a second power supply before installation.

Quantities exclude spares, mounting accessories, patch leads and licensing unless stated. Prices are provided separately.

# 7. Network design

## Wireless networks (SSIDs)

| SSID | VLAN | Security | Authentication | Bands | Notes |
| --- | --- | --- | --- | --- | --- |
| CORP | 10 | WPA3-Enterprise | 802.1X / RADIUS | 5 + 6 GHz | Staff devices |
| GUEST | 20 | WPA2-Personal + captive portal | Portal | 2.4 + 5 GHz | Internet only |
| IOT | 30 | WPA2-PSK | PSK | 2.4 GHz | Sensors, printers |

## VLANs and addressing

| VLAN | Name | Subnet | Gateway | DHCP |
| --- | --- | --- | --- | --- |
| 10 | Corporate | 10.10.10.0/24 | 10.10.10.1 | Central DHCP |
| 20 | Guest | 10.10.20.0/23 | 10.10.20.1 | Gateway DHCP |
| 30 | IoT | 10.10.30.0/24 | 10.10.30.1 | Central DHCP |
| 99 | AP management | 10.10.99.0/24 | 10.10.99.1 | Static / DHCP |

## Supporting services

| Service | Address |
| --- | --- |
| RADIUS / NAC | 10.10.5.10 / 10.10.5.11 |
| DHCP | 10.10.5.20 |
| DNS | 10.10.5.20, 10.10.5.21 |
| NTP | ntp.internal |
| Syslog / monitoring | 10.10.5.30 |

# 8. Physical connectivity

How the access points connect back to the switching, the controller and the internet.

Access point to access switch to controller / gateway topology

# 9. Logical design

How each wireless network maps to a VLAN, a gateway and the services behind it.

SSID to VLAN to service mapping

# 10. Findings and observations

| Ref | Location | Category | Observation | Severity | Recommendation |
| --- | --- | --- | --- | --- | --- |
| F-01 | Service core | Coverage | The concrete core sits in the middle of the floor plate and blocks the direct path between the north and south halves. | Medium | Keep APs on both sides of the core - the layout in this report already does. |

# 11. Risks, assumptions and dependencies

| Ref | Type | Description | Impact | Likelihood | Mitigation / owner |
| --- | --- | --- | --- | --- | --- |
| R-01 | Risk | Asbestos register lists material in the plant room; that area was not surveyed. | High | Medium | Client to supply the full register before installation - J. Client |

# 12. Actions and next steps

| Ref | Action | Owner | Due | Status |
| --- | --- | --- | --- | --- |
| A-01 | Confirm cable routes and containment with the FM team | Contoso | 2026-10-05 | Not started |
| A-02 | Order the hardware in the bill of materials | Niyyasoft | 2026-10-12 | Not started |

# 13. Conclusions and recommendations

| Item | Detail |
| --- | --- |
| Survey summary | Eleven Wi-Fi 6E access points cover 100 % of the 1,302 m2 floor plate at or above -67 dBm. |
| Recommendations | Adopt the eleven-AP layout |

# 14. Sign-off

| Item | Detail |
| --- | --- |
| Surveyor | A. Engineer |
| Date | 2026-09-21 |

# Appendix A - Method and assumptions

- Coverage is predictive: it is modelled from the floor plan, the wall materials drawn on it, and the vendor radio specifications - it is not a measured (AP-on-a-stick) survey.

- Path loss uses the log-distance model with per-wall attenuation added along the direct path between AP and measurement point.

- Transmit power and antenna gain come from the vendor data sheets and must be re-checked against the regulatory domain of the installation.

- Scale accuracy governs everything else - confirm the plan scale with a known on-site dimension before construction.

- Validate the design with a post-installation survey before hand-over.

## Model settings used

| Setting | Value |
| --- | --- |
| Environment | office |
| Path-loss exponent | 3 |
| Noise floor | -92 dBm |
| Primary coverage target | -67 dBm |
| Secondary coverage target | -72 dBm |
| Assumed client transmit power | 15 dBm |
| Fade margin | 3 dB |
| Grid resolution | 6 px |

# Appendix B - Glossary

Terms used in this report, in plain language.

| Term | What it means |
| --- | --- |
| Access point (AP) | The device on the ceiling or wall that provides the wireless signal. |
| dBm | The unit of signal strength. Always negative: -50 dBm is strong, -80 dBm is weak. |
| RSSI | Received signal strength - how loudly a device hears an access point. |
| SNR | Signal to noise ratio, in dB. How far the signal stands above background noise. Above 25 dB is comfortable; below 15 dB performance falls away. |
| Channel | The slice of radio spectrum an access point uses. Neighbours on the same channel share airtime, so channels are planned to keep them apart. |
| Band | 2.4 GHz travels furthest but is slow and congested. 5 GHz is the workhorse. 6 GHz is the newest, fastest and cleanest, and needs Wi-Fi 6E or Wi-Fi 7 devices. |
| Wi-Fi 6 / 6E / 7 | Generations of the standard. Each adds capacity; 6E and 7 add the 6 GHz band. |
| SSID | The wireless network name a user picks from the list on their device. |
| VLAN | A separate logical network on the same physical cabling, used to keep guest, corporate and device traffic apart. |
| PoE | Power over Ethernet - the network cable also powers the access point, so no local socket is needed. 802.3af, at and bt supply progressively more power. |
| Controller | The appliance or cloud service that configures and monitors every access point. |
| Roaming | Moving between access points without dropping a call or a session. |
| Co-channel interference | Two nearby access points on the same channel taking turns, which halves the airtime available to each. |
| DFS | Channels shared with radar. Access points must vacate them if radar is detected. |
| Predictive survey | Coverage modelled from drawings and materials, as in this report. |
| Validation survey | Coverage measured on site after installation, to prove the design. |

# Document information

| Item | Detail |
| --- | --- |
| Reference | NSS-2026-042 |
| Revision | A |
| Status | For client review |
| Produced with | WiFi Site Survey Studio by Niyyasoft |
| Report completeness | 16 of 101 fields (16 %) |